# CAN Decode Application Readme

To run application:

Open main.py and edit the video and CAN data files to the correct file and path. Then run the file.

NOTE: The CAN data file needs to be in the correct format with the correct column names as shown in the example for the code to work.

## Variables of Interest:

### CAN_details:

This variable shows the CAN Id, and the number of bytes in each can ID. It also shows the number of times each byte has changed. This is a Pandas dataframe with fields:

- ID	           : CAN ID

- Msg_Length: Message length in bytes

- No	           : Number of times this CAN ID was published

- 0…(Msg_Length-1): This field shows how many times each of the bytes in the message changed.

Use this dataframe to check which CAN Id/Byte combo is of interest depending on how many times it changed.

### Candidates:

This analyzes the bytes that changed in CAN_details bit by bit to show how many times each bit in the byte has changed. This is a Pandas Dataframe with fields:

- Index, index: ignore this field

- ID 	: CAN ID

- Msg_Length: Length of the message in bytes

- No 	: number of times this CAN Id was published

- Byte No: The byte that has been analyzed in this row

- No. of changes: The number of times this byte has a whole has changed

- 0…7: Each column here denotes the number of times that particular bit has changed.

  - NOTE: the columns are numbered wrong, it should be 7 to 0 instead of 0 to 7 as that is the convention in CAN data. This needs to be fixed in future edits to the code.

## Using the GUI:

To run the GUI, run the last block of code:

#### GUI:

- CAN ID: enter CAN Id to be plotted.

- Bytes List: enter the bytes to be plotted in a list format, in the order to be concatenated.

- Bits list of list: enter the bits for each byte to be concatenated.

Example:

In the example in the figure:

- Bytes list = [2,1],

- Bits list of list = [[0,7],[0,7]]

This will concatenate byte 2 and byte 1 in that order, and all 8 bits ([0,7]) of byte 2 will be concatenated with all 8 bits ([0,7]) of byte 1 and plotted.